.VERSION 1.0

.DATA_SOURCE    Technical_University_of_Darmstadt

.DATA_ID        dar2

.HIERARCHY      PAGE TEXT WORD

.DATA_CONTACT

Name: Jan Sendler

Affiliation: Technical University of Darmstadt, Depart. Datentechnik (Digital Engineering)

Address: Merckstrasse 25

Phone: +49 6151 16-2078

Fax: +49 6151 16-4976

Email: jan@dtro.e-technik.th-darmstadt.de

.SETUP    Site: office

Time: 09/23/94 - 09/29/94

Writer population:

Writer motivation: volunteer

Writer physical position: sitting

Instructions given to the writer:  	The writer copies

the paragraph displayed on

the screen in front him.

He should notice the horizontal

guide lines.

Duration of one session: 1 h

Recognizer feedback: no

Form layout: A4 paper sheet,

7 equidistant horizontal guidelines

distance 25mm, width 190mm.

postscript file:

.DATA_INFO

Set type: training

Alphabet: English keyboard alphanumerics and symbols

from the ASCII set

Lexicon: iacocca

Label distribution: manually

Quantity: ~ 13000 char

Quality: cleaninig/no cleaning

Writer style: cursive/mixed

Segmentation: manually

.PAD     Machine name: CALCOMP DrawingBoard II

Brand: CalComp

Type: Mod 33120

Serial Nr.: ZAA29614

Sensor: Electromagnetic, pressure-sensitive, wireless pen

Pen: Ballpoint pen, zero-pressure is used for PenUp determination.

Driver: Home made driver using "/dev/tty" on SparcStation running Solaris 2.3.

Sampling mode: Equidistant in time, between .PenDown and .PenUp.

Sampling rate: 200 [Hz]

Resolution: 0.01 [mm/unit]

Accuracy: 0.254 [mm]

Display: Paper, Ink

Inking: black on white, trajectory of ballpoint.

.X_DIM 30000

.Y_DIM 30000

.H_LINE 20000 17500 15000 12500 10000 7500 5000

.X_POINTS_PER_MM 100

.Y_POINTS_PER_MM 100

.POINTS_PER_SECOND 200

.POINTS_PER_GRAM 1000

.COORD X Y P